Example Risk Assessment

The following is an example of risk assessment for a HMO that would be subject to mandatory licensing or a house or small block with shared common parts. The example is not exhaustive and is intended to give the responsible person an idea of what a fire risk assessment could look like. A blank fire risk assessment is reproduced at the end of this guide that you may like to use.

| Risk Assessment - Record of significant findings |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Risk assessment for Building: 23 Hammersmith Grove Hammersmith London W6 |  |  | Assessment undertaken by Date: 22 April 2010 Completed by: John Smith Signature: J Smith |  |  |
| Sheet number Floor/area: Three storey semi detached house, with five bedrooms, bathroom, WC, & kitchen |  |  | Use: Residential property occupied by five individuals. |  |  |
| Step 1 - Identify fire hazards |  |  |  |  |  |
| Sources of ignition Cooking Smoking materials Electrical Installation Gas Installation Candies |  | Sources of fuel Furniture Floor & wall coverings Polystyrene ceiling tiles Accumulation of junk mail |  |  | Sources of oxygen None other than naturally occurring |
| Step 2 - People at risk |  |  |  |  |  |
| Residents and visitors to the property |  |  |  |  |  |
| Step 3 - Evaluate, remove, reduce and protect from risk |  |  |  |  |  |
| What is the risk? Who is at risk? Action taken to remove and reduce the hazards that may cause a fire Action taken to remove and reduce the risk to people from a fire | Risk from cooking higher than single family home as each resident cooks independently. Smoking permitted in the house Fire can spread throughout the property and all tenants and guests at risk, particularly at night when asleep. Smoking banned within the property Candles banned within the property Junk mail re-cycled Polystyrene ceiling tiles removed and replaced Electrical Installation tested for safety Gas Installation tested for safety Furniture replaced to comply with fire regulations where necessary AFD Grade A LD2 installed Tenants told how to use fire blanket in kitchen and extinguishers. Tenants informed not to interfere with self closers on doors. 3 self closers adjusted so doors close properly |  |  |  |  |
| Assessment review Assessment/ review date | Completed by |  |  | Signature |  |